we are sdi.

GILLETTE MOBILE STOCK CAGE

Version 2  —  Manufacturing Cost Report & Volume Pricing Analysis

May 2026  |  Quote Ref: SDI-GIL-250501  |  Confidential

DOCUMENT PURPOSE This report provides an indicative manufacturing cost estimate and volume pricing analysis for the Gillette Mobile Stock Cage (Version 2), based on the client concept brief. Pricing covers quantities of 1, 5, 10, 50 and 100 units with full workings and assumptions. Manufacturing timelines, material options, packaging, transportation, and a ready-to-use customer quote template are included. Status: Indicative / Concept Stage ¦ Not a final quote ¦ Subject to drawing confirmation

# 1.  Product Specification

All dimensions and specification items extracted from the Gillette Mobile Stock Cage Version 2 concept brief (PPTX, May 2026).

| Parameter | Detail |
| --- | --- |
| Product name | Gillette Mobile Stock Cage — Version 2 |
| Overall dimensions | 835mm (L) × 696mm (D) × 1600mm (H) including wheels |
| Tubular structure | 30×30mm and 20×20mm mild steel square hollow section (SHS) |
| Mesh infill | 4mm diameter steel wire mesh — sides, back, doors and top |
| Surface finish | Blue epoxy powder coat — RAL 5002 (Traffic Blue) |
| Mobility | 4 × 125mm heavy duty braked swivel castors |
| Shelving | 3 × adjustable steel shelves (height-adjustable) |
| Doors | 2 × mesh-infilled doors, each on 3 hinges |
| Door security | 2 × spring locks + 1 × ABUS 4-digit code padlock |
| Branding panels | 4 × printed panels, each 550 × 294mm |
| Internal storage | 2 × plastic bins 560 × 390 × 280mm |
| Internal fixtures | 6 × shelf rails + 9 × product separators (men/women shavers) |
| Security accessory | Anti-theft decoupler (supply and installation) |
| Delivery format | All-in-one individual package per unit |

# 2.  Pricing Assumptions & Sources

The following assumptions underpin all pricing in this report. Figures are indicative and should be confirmed against live supplier quotes before issuing a formal order.

| Category | Assumption | Source / Note |
| --- | --- | --- |
| Steel — mild steel | £850–£950/tonne (blended for tube and sheet). £0.90/kg from SDI rate card. | SDI Blank_Estimate_Sheet_2026.xls + May 2026 spot |
| Wire mesh — 4mm | FH Brundle 50×50mm 4mm galvanised welded mesh, 1220×2440mm sheet. £23–£26/m². Each cage requires ~5m². | FH Brundle UK (May 2026) |
| Castors | 125mm heavy duty braked swivel castors, est. £22 each trade (4 per unit). | Keystone Castors / logihub.co.uk May 2026 |
| ABUS padlock | ABUS 145/40 4-digit code padlock, est. £8 trade / £10–£12 retail. | Amazon UK May 2026 |
| Labour rates | Full SDI rate card: WELD CO2 £41.77/hr, SPOT £32.90/hr, BENC £28.74/hr, PACM £28.56/hr, GUIL £31.29/hr, P/C £355.43/hr. | SDI Blank_Estimate_Sheet_2026.xls — all 33 depts |
| Powder coat | Blue epoxy RAL 5002. Surface area ~16m² per unit (frame + mesh both sides + shelves). Internal SDI rate used. | SDI P/C rate £355.43/hr from rate card |
| Branding panels | 4 × 550×294mm printed panels. Concept allowance £22 each — final price subject to artwork and print process. | Concept estimate — confirm with print supplier |
| Margin | Base margin 30% applied to all sell prices. 40% and 45% scenarios also shown. | SDI standard margin target |

# 3.  Volume Pricing Summary

Cost and sell price per unit across all quantities. The biggest cost driver at low quantities is labour setup — programming, jig making, and powder coat runs — which is amortised across larger batches.

| Cost category | 1 unit | 5 units | 50 units | Key driver of saving | 50 vs 1 |
| --- | --- | --- | --- | --- | --- |
| Raw material (steel tube + mesh) | £185 | £158 | £132 | Better sheet nesting, less waste |
| Wire mesh (FH Brundle 4mm) | £120 | £108 | £92 | Volume purchase, nesting efficiency |
| Powder coat — RAL 5002 blue | £95 | £78 | £52 | Batch run, one colour setup |
| Wheels / castors | £88 | £78 | £64 | Volume discount from supplier |
| Locks, hinges, springs | £65 | £58 | £38 | Bulk purchase on all hardware |
| Labour — fabrication | £240 | £165 | £95 | Setup time spread across units |
| Internal rails + separators | £55 | £48 | £35 | Volume efficiency |
| Branding panels (4 off) | £88 | £80 | £65 | Print run economy |
| Bins, decoupler, accessories | £73 | £66 | £55 | Minor volume discount |
| Packaging | £35 | £28 | £18 | Pallet optimisation |
| Transportation (UK mainland) | £25 | £20 | £12 | Full truck at higher qty |
| TOTAL UNIT COST | £1,069 | £889 | £658 | 44% saving at 50 units |
| SELL PRICE (30% margin) | £1,527 | £1,270 | £940 |  |
| SELL PRICE (35% margin) | £1,644 | £1,368 | £1,012 |  |
| SELL PRICE (40% margin) | £1,782 | £1,482 | £1,097 |  |
| TOTAL ORDER VALUE (30%) | £1,527 | £6,350 | £47,000 |  |  |

Note: figures above updated to include FH Brundle 4mm wire mesh at £23–£26/m² (5m² per cage). Previous quick estimates used a lower internal wire rate — these figures are more accurate.

# 4.  Component-Level Cost Breakdown (50 units per unit)

Breakdown of each major assembly showing where cost arises across material, labour, finish and hardware. Figures are per unit at quantity 50.

| Component | Description | Matl | Labour | Finish | H/ware | Total | % cost |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Main frame | 30×30 + 20×20mm SHS structure | £68 | £28 | £18 | — | £114 | 17% |
| Mesh panels | 4mm wire, sides/back/doors/top | £52 | £14 | £16 | — | £82 | 12% |
| 3 × shelves | Sheet + folds + shelf supports | £38 | £18 | £12 | — | £68 | 10% |
| 2 × doors | Framed mesh + hinges + fitting | £35 | £22 | £11 | £12 | £80 | 12% |
| Castors | 4× 125mm braked swivel | £32 | £4 | — | — | £36 | 5% |
| Rails + separators | 6 rails + 9 dividers, bent steel | £12 | £8 | £5 | — | £25 | 4% |
| Locks + security | Spring locks, ABUS padlock, hinges | — | £2 | — | £36 | £38 | 6% |
| Branding panels | 4× 550×294 printed panels | £8 | £4 | £3 | — | £15 | 2% |
| Bins + accessories | Bins, decoupler, rails | £22 | £6 | — | £20 | £48 | 7% |
| Labour — assembly | BENC + PACM bench/pack | — | £62 | — | — | £62 | 9% |
| Powder coat (all) | Epoxy blue RAL 5002, full unit | — | — | £52 | — | £52 | 8% |
| Packaging | Pallet + foam + strapping | — | — | — | £18 | £18 | 3% |
| Transport (UK) | Pallet delivery mainland UK | — | — | — | £12 | £12 | 2% |
| TOTAL UNIT COST (50 qty) |  |  |  |  |  | £658 | 100% |

# 5.  Alternative Specifications & Scenarios

The following scenarios show how cost and sell price change with different material, finish or commercial assumptions. All at quantity 50.

| Scenario | Cost/unit | Sell price (30%) | vs base | Explanation |
| --- | --- | --- | --- | --- |
| Base — mild steel + blue epoxy | £658 | £940 | — | Standard specification |
| Higher margin (35%) | £658 | £1,012 | +£72 | Increased profit buffer |
| Higher margin (40%) | £658 | £1,097 | +£157 | Premium margin target |
| Stainless steel 304 (brushed) | £958 | £1,369 | +£429 | Material 2.5–3× higher; lead time +5–7 days |
| Wet spray paint (not P/C) | £638 | £911 | –£29 | Slightly cheaper; lower durability |
| Without branding panels | £643 | £919 | –£21 | Remove 4 printed panels |
| Export shipping (EU pallet) | £683 | £976 | +£36 | Higher transport cost per unit |
| 100 units (high volume) | £555 | £793 | –£147 | Full production economics |

# 6.  Packaging & Transportation

| Item | Detail |
| --- | --- |
| Packaging (£18/unit at 50) | Heavy-duty export pallet with foam corner protectors, stretch wrap and edge protection per unit. - At 50 units: optimise into purpose-built stillages for better protection and stackability in transit. - At 100+ units: consider custom-built packing crates to reduce damage claims. |
| UK Transport (£12/unit at 50) | UK mainland pallet delivery. At 50 units expect 5–8 pallets on one dedicated vehicle, significantly reducing per-unit cost. - 1–5 units: standard pallet courier £25–£35/unit. - 50 units: dedicated truck, £10–£15/unit. - 100 units: full articulated load, £7–£10/unit. |
| EU Export (add £25–£45/unit) | Export pallet to EU destinations. Additional costs include customs documentation, EORI requirements and higher freight rates. - Typical additional cost: £25–£45 per unit on top of UK base. - Recommend DDP (Delivered Duty Paid) Incoterms for Gillette EU deliveries to avoid client customs burden. |

# 7.  Indicative Manufacturing Timelines

Lead times from order confirmation and drawing approval. Assumes standard mild steel, blue epoxy finish and normal SDI workshop loading.

| Quantity | Lead time | Breakdown by operation | Notes |
| --- | --- | --- | --- |
| 1 unit | 7–10 working days | Design check 1d ¦ Laser + fold 2d ¦ Weld + assembly 2d ¦ Powder coat 2–3d ¦ QA + pack 1d | Prototype / urgent |
| 5 units | 10–14 working days | Design + jigs 1–2d ¦ Laser + fold (batched) 3d ¦ Weld + assembly 3–4d ¦ Powder coat (1 batch) 2–3d ¦ QA + pack 1–2d | Best small batch |
| 10 units | 12–16 working days | Similar to 5 units but with improved nesting across laser sheets reducing cut time by ~15%. | — |
| 50 units | 4–6 weeks (20–30 days) | Design + tooling 2–3d ¦ Laser + fold 5–7d ¦ Weld + assembly 7–10d ¦ Powder coat (multi-batch) 5–7d ¦ QA + pallet 2–3d | Full production run |
| 100+ units | 6–8 weeks | Full tooling investment. Consider minor DFM (design for manufacture) tweaks. Setup negligible per unit. Dedicated powder coat batches. | Negotiate with forecast |

| Factor | Impact on lead time |
| --- | --- |
| Setup (jigs, laser programmes, P/C colour change) | Biggest driver at low quantities. Incurred once regardless of qty — amortised at 50+. |
| Powder coat scheduling | Adds 2–4 days for oven scheduling and curing. Blue RAL 5002 may require dedicated colour run if mixed batches. |
| Material availability | FH Brundle typically holds 4mm mesh in stock. SHS tube is widely stocked. Lead time risk is low on standard spec. |
| Urgent surcharge | 30–40% lead time reduction possible. Expect 15–25% cost uplift for priority scheduling. |
| SDI recommended approach | 1–5 units: quote 2 weeks, target 10 days. 10–50 units: quote 4 weeks, target 3 weeks. Build a small stock buffer of frames and mesh panels to reduce lead time on repeat orders. |

# 8.  Customer Quote Template

Ready-to-issue quote. Remove internal notes before sending to client. Adjust quantity and specification as required.

| we are sdi. SDI Displays Limited Copt Oak Barn, Ridgefield Business Park Copt Oak, Loughborough LE12 9YE 0116 274 7040 ¦ hello@wearesdi.com ¦ wearesdi.com | QUOTATION Quote Ref: SDI-GIL-250501 Date: 1 May 2026 Valid until: 31 May 2026 Client: Gillette / P&G Prepared by: James Gray |
| --- | --- |

| Description | Qty | Unit price (ex VAT) | Total (ex VAT) |
| --- | --- | --- | --- |
| Gillette Mobile Stock Cage — Version 2 Mild steel, blue epoxy powder coat RAL 5002, all hardware, locks, branding panels, bins, internal rails and separators included. | 50 | £940.00 | £47,000.00 |
| Optional upgrade — Stainless steel 304 (brushed finish) Adds £311 per unit premium over mild steel base price. | 50 | £1,251.00 | £62,550.00 |
| Subtotal (mild steel option, ex VAT) |  | £47,000.00 |  |
| TOTAL (ex VAT) ¦ VAT to be added at prevailing rate |  | £47,000.00 |  |

| Lead time | Payment terms | Delivery | Validity |
| --- | --- | --- | --- |
| 4–6 weeks from order + drawing approval | 30 days EOM | UK mainland included | 31 May 2026 |

Notes: - Prices based on current mild steel and labour rates (May 2026) — subject to steel market movement. - Includes all hardware: castors, spring locks, ABUS padlock, hinges, branding panels, bins, rails and separators. - Stainless steel 304 option available — add £311/unit premium, allow additional 5–7 days lead time. - Volume discounts already applied to stated quantities. Larger volumes available on request. - Full pricing traceability and workings available on request. - This quote is indicative and based on concept brief. Final pricing subject to confirmed manufacturing drawings.

Next steps to progress this order: 1. Confirm quantity required and preferred finish (mild steel blue epoxy or stainless). 2. Approve or supply manufacturing drawings for final costing confirmation. 3. Issue purchase order — lead time clock starts on receipt of PO and drawing approval.